| Site Location: | Project #: | Jurisdiction: ☐Federal ☐WA State | Soil Type: Type C |
| --- | --- | --- | --- |
| Description of Work: |  |  |  |
| A supervisor/competent person shall make daily safety inspections of excavations/trenches, adjacent areas, and protective systems for evidence of situations that could result in possible cave-ins, indications of failure of protective systems, hazardous atmospheres, and other hazardous conditions. The supervisor/competent person shall conduct inspections daily and after other potential hazard increasing occurrences (heavy rain, etc.). These inspections are required only when worker exposure can be reasonably anticipated in excavations over 4 feet (Washington DOSH) and 5 feet (Federal OSHA) when occupied by workers. |  |  |  |
| COMPLETE CHECKLIST AT THE BEGINNING OF EACH SHIFT |  |  |  |

| Site Assessment |  |  |  | Shielding/Shoring System |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Utilities marked/verified, protected as needed? |  |  | ☐YES ☐N/A | Shoring equipment inspected and in good condition? |  |  | ☐YES ☐N/A |
| Traffic control plan needed? |  |  | ☐YES ☐N/A | Excavation is <2 feet below the bottom of the support system? |  |  | ☐YES ☐N/A |
| Area around excavation protected & barricaded? |  |  | ☐YES ☐N/A | Support system is stable/secure? |  |  | ☐YES ☐N/A |
| Water accumulation controlled? |  |  | ☐YES ☐N/A | Support system installed per manufacturers specifications? |  |  | ☐YES ☐N/A |
| Sloping system checked for evidence of failure (soil distress, fissures, structural damage, etc.)? |  |  | ☐YES ☐N/A | Where there is exposure on an open end of a support system, sloping, plates or panels are used to prevent cave-ins? |  |  | ☐YES ☐N/A |
| Confined space evaluation where applicable? |  |  | ☐YES ☐N/A | Support system extends at least 18” above the top of the excavation? |  |  | ☐YES ☐N/A |
| Employees briefed on associated hazards? |  |  | ☐YES ☐N/A | Safe access/egress is provided (no more than 25 feet away)? |  |  | ☐YES ☐N/A |
| General Precautions |  |  |  |  |  |  |  |
| Adequate boundaries/barricading have been established? |  |  | ☐YES ☐N/A | Foundations, structures, appurtenances, etc. have been evaluated/supported? |  |  | ☐YES ☐N/A |
| Ladders extend 36” above top of protective system? |  |  | ☐YES ☐N/A | Walkways/bridges with standard guardrails provided where employees are required to cross the excavation/trench? |  |  | ☐YES ☐N/A |
| Spoils kept at least 2ft. back from the edge of excavation? |  |  | ☐YES ☐N/A | Any changes to scope, site conditions or hazards reviewed with employees? |  |  | ☐YES ☐N/A |
| Surcharge loads evaluated for potential sidewall failure? |  |  | ☐YES ☐N/A | Thrust blocks/mechanical restraints verified in place (hydrants, water main T’s, Y’s, 90’s and 45’s) |  |  | ☐YES ☐N/A |
| Protective System Used (select all that apply) |  |  |  | Shielding/Shoring System Used (Select all that apply) |  |  |  |
| Shielding/Shoring |  |  | ☐ | ☐ Hydraulic Shoring | ☐ Trench Boxes |  |  |
| Sloping (1 ½ : 1 minimum) |  |  | ☐ | ☐ Speed Shoring | ☐ Engineered System |  |  |
| Engineered System (>20ft. in depth requires design from professional engineer. Maintain plans on site.) |  |  | ☐ | Other (describe): |  |  |  |
| Comments: |  |  |  |  |  |  |  |
| DATE | TIME | COMMENTS/ISSUES | CORRECTIVE ACTION |  | COMPETENT PERSON | Safe to Enter |  |
|  |  |  |  |  |  | ☐YES ☐NO |  |
|  |  |  |  |  |  | ☐YES ☐NO |  |
|  |  |  |  |  |  | ☐YES ☐NO |  |
|  |  |  |  |  |  | ☐YES ☐NO |  |
|  |  |  |  |  |  | ☐YES ☐NO |  |
|  |  |  |  |  |  | ☐YES ☐NO |  |
|  |  |  |  |  |  | ☐YES ☐NO |  |
|  |  |  |  |  |  | ☐YES ☐NO |  |