# Building Walkthrough Notetaking Checklist

Refer to the “Building Walkthrough Checklist with Guidance” document for detailed information for each checklist item.

BUILDING NAME/NUMBER/TYPE: ______________________________________________

## General Building Review

☐ Building occupancy schedule

☐ Building pressure (positive/negative)

☐ Hot/cold spots

## Airside Equipment

### Components

☐ Count and configuration of equipment

### Operations

☐ Review equipment H-O-A switch positions

☐ Review VFD speed outputs

### Dampers, valves, actuators

☐ Examine damper seals

☐ Examine linkages

☐ Review damper positions

☐ Examine valves (disconnected actuators, isolation valves closed, etc.)

### Heating and cooling coils

☐ Evaluate heating/cooling coil valve positions

☐ Use IR gun or thermal imaging camera to identify leaking valves

☐ Review valves for hunting behavior

Notes:

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Zone Level Review

☐ Review zones HVAC design (VAVs, fan powered boxes, perimeter heating, etc.)

☐ Review thermostats in the zones and note features

### Fans

☐ Nameplate data for fan motor (if mechanical schedules not up to date)

### General condition

☐ Review equipment for physical issues

☐ Check for dirty filters and coils

### Sensors

☐ Confirm location for temperature and humidity sensors and values

☐ Supply air

☐ Return air

☐ Mixed air

☐ Outside air

### Freeze mitigation (if applicable)

☐ Review freeze mitigation strategy

☐ Note freezestat setpoint

## Heating and Cooling

## Cooling System

☐ Note cooling system details

☐ Equipment count

☐ Equipment rating/capacity

☐ Equipment type

☐ Speed control

☐ Equipment configuration and valves

### Pumps

☐ Review equipment H-O-A switch positions

☐ Review VFD speed outputs

☐ Evaluate if pumps are throttled

☐ Observe if pumps are running when associated boiler/chiller is off

### Chiller

☐ Chilled water (ChW) (supply/return) temperatures

☐ Condenser water (supply/return)     temperatures

☐ Evaluate chiller/pump staging

☐ Evaluate cooling tower fan speeds

☐ Check local display for ChW setpoint control (local/remote)

☐ Check local display for alarms

## Heating System

☐ Note heating system details

☐ Equipment count

☐ Equipment rating/capacity

☐ Equipment type

☐ Speed control

☐ Equipment configuration and valves

### Insulation

☐ Examine insulation (missing, sweating pipes)

### Valves

☐ Verify position of manual valves

☐ Verify position of isolation valves on chillers and boilers

### Boiler

☐ Note hot water (HW) temperatures (supply/return)

☐ Evaluate boiler/pump staging

☐ Check local display for HW setpoint control (local/remote)

☐ Check local display for alarms

Notes:

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________